Pennsylvania Department of Environmental Protection

Bureau of Abandoned Mine Reclamation

Acid Mine Drainage Division

Cambria Office

Upper Blacklick Creek Biological Assessment (USGS HUC 05010007000137)

Prepared by

Joseph Cocco

Dates Sampled:  April 2015, August 2015 and November 2017

Introduction

Blacklick Creek is a 51.9 kilometer (km) stream that flows from its source northwest and south of Ebensburg, PA, in Cambria County and southwest to its confluence with Conemaugh Lake near Saltlick, PA.  Blacklick Creek has two (2) major tributaries, North Branch and South Branch, both large sections of the watershed.  The watershed is 1085.7 km2.  The majority of the region is rural and forested; however, many rural and small-town businesses are located throughout the area.  Yellow Creek State Park is in the watershed and is one of the most popular state parks in central PA.  Several streams are stocked in the Blacklick Creek watershed annually by the PA Fish and Boat Commission (PFBC) and small co-op hatcheries.  There are several special regulations within the watershed including preseason closing for trout-stocked sections and panfish enhancement regulations on Yellow Creek Lake.

The Pennsylvania Department of Environmental Protection (PA DEP) Chapter 93 designation for Blacklick Creek is Trout Stock Fishery (TSF) (PA DEP 1999).  The North Branch and the South Branch of Blacklick Creek are listed as Cold Water Fisheries (CWF).  Acid Mine Drainage (AMD) has been a major factor in causing water quality degradation in the Blacklick Creek Watershed.  Blacklick Creek Watershed has many passive treatment sites and many untreated discharges from the headwaters all the way downstream to the confluence with Conemaugh Lake.  Elk Creek, a tributary of North Branch Blacklick Creek, is the first major source of AMD pollution affecting the North Branch several miles away from the Redmill Discharge.  Redmill Discharge and the Three Sisters Discharge are located near the lower section near the mouth of the North Branch and severely impact the watershed.  South Branch has several mining areas with small discharges from the headwaters down to Twins Rocks, PA, including the Webster Discharge near Nanty Glo.  The most influential discharge is the Vinton No. 3 Discharge and corresponding refuse coal pile.  AMD also affects the main stem of Blacklick Creek.  The Wehrum Discharge is the first and largest discharge that sets the tone for impairment down to the confluence with the Conemaugh River Reservoir.  PA DEP Bureau of Abandoned Mine Reclamation (BAMR) is currently developing a plan for the construction of an active treatment plant that would tie in several mine pools, eliminating the previous mentioned discharges located near Vintondale, PA.  The active treatment will substantially increase water quality and has the potential to drastically change fishing opportunities.

The purpose of this survey was to:  1) compile a comprehensive water quality and biological data summary for the Upper Blacklick Creek Watershed pertaining to the Vintondale AMD Treatment Plant, 2) assess water quality through the occurrence of macroinvertebrates for Index of Biotic Integrity (IBI) and relative abundance of Blacklick Creek fishes, 3) use data to update ongoing management plans for AMD treatment on Blacklick Creek, 4) Collect and enumerate all fish species for future developed IBI and 5) provide the data to organizations involved with water quality and recreational improvement in the Blacklick Creek Watershed.

Methods

Nine (9) macroinvertebrate sites and six (6) electrofishing sites were surveyed in 2015 and 2017. Each site was chosen to help provide an overview of the entire watershed and effects on the receiving water.

Nine (9) macroinvertebrates sites were sampled during April 2015 and November 2017.  Site 1 - North Branch Blacklick Creek Upstream Redmill Discharge (NBBLCUSRMDG) was located at Latitude 41.5125, Longitude 78.9036.  Site 2 - North Branch Blacklick Creek Downstream Redmill Discharge (NBBLCDSRMDG) was located at Latitude 40.5048, Longitude 78.9049.  Site 3 - North Branch Blacklick Creek Upstream Three Sisters Discharge (NBBLCUSTS) was located at Latitude 40.4874, Longitude 78.9205.  Site 4 - North Branch Blacklick Creek Mouth (NBBLCM) was located at Latitude 40.4840, Longitude 78.9238.  Site 5 - South Branch Blacklick Creek Twin Rocks (SBBLCTR) was located at Latitude 40.4904, Longitude 78.8628. Site 6 - South Branch Blacklick Creek Mouth (SBBLCM) was located at Latitude 40.4837, Longitude 78.9238.  Site 7 - Blacklick Creek Downstream Confluence North and South Branches (BLCDSSB) was located at Latitude 40.4817, Longitude 78.9311.  Site 8 - Blacklick Creek Upstream Wehrum Discharge (BLCUSWDG) was located at Latitude 40.4703, Longitude 78.9465.  Site 9 - Blacklick Creek Downstream Wehrum Run (BLCDSWR) was located at Latitude 40.4837, Longitude 78.9238.

Kick sampling was conducted in accordance with the protocols from Standardized Biological Field Collection and Laboratory Methods (PA DEP 2003).  Samples were then processed for identification using the protocol from the Instream Comprehensive Evaluation Surveys (ICE) adopted by PA DEP (PA DEP 2013).  ICE uses several macroinvertebrate Indices of Biotic Integrity (IBI) such as Taxa Richness, EPT Taxa Richness, Beck’s Index, Hilsenhoff Biotic Index, Shannon Diversity and Percent Sensitive Individuals.  Each index represents the relative health of the stream by measuring different aspects of macroinvertebrate biotic health.  They are then standardized to compute the Aquatic Life Use (ALU) IBI score.  The ALU is used to rate streams as impaired or unimpaired (PA DEP 2013).  The unimpaired benchmark must be equal to or greater than 63 to qualify as attaining ALU.  ALU is also computed two ways:  Small Freestone Stream (SFS) and Large Freestone Stream (LFS).  Each one tests the ALU standardizations in a way that represents the change in macroinvertebrate communities between small streams and larger streams/small rivers.  This is a useful tool when describing AMD affected waters in the recovery process.  Habitat assessments were conducted at each site in accordance with PA DEP ICE procedures (PA DEP 2013), which are similar to Procedures for Stream and River Inventory Information Input (Marcinko, et al. [1986]).

Five (5) electrofishing sites were sampled in late August 2015 and November 2017.  All fish were collected and enumerated to be used in calculating an Index of Biotic Integrity (IBI).  Currently an official IBI for Pennsylvania is under development.  All electrofishing sites correspond with the same macroinvertebrate site as follows:  NBBLCUSRMDG, NBBLCUSTS, BLCUSWDG, SBBLCTR and SBBLCM.

Electrofishing was conducted using the protocols from Standardized Biological Field Collection and Laboratory Methods (PA DEP 2013, re. 2013).  Section VI – Fishes was updated in 2013.  Each site was surveyed with an Aqua Shock Solutions B-1-L unit, which utilizes a two-active probe system.  This unit does not record amps or watts, and current ranged from 75 - 125 volts

AC at all sites.  All sites were conducted with AC current to maximize fish capture.  The lowest possible voltage was used to ensure fish survival.  All sites were 200 meters (m)  in length and encompassed multiple habitat types.

Results

North Branch Blacklick Creek Upstream Redmill Discharge (NBBLCUSRMDG) located at river kilometer (RK) 5.75 km, was sampled for macroinvertebrates in April 2015.  Habitat is optimal with a total score of 223 (Table 18A).  Field chemistry conditions of the water were:  9.3o C, Dissolved Oxygen was 10.0 mg/L, pH was 7.3 and Alkalinity was 1.0 mg/L. Macroinvertebrates assemblage is located on Table 2.  Taxa Richness was 15 and EPT Taxa Richness was 9.  The Becks Index was 7 and the Percent Sensitive Individuals was 32.81%. Hilsenhoff Biotic Index was 4.59, while the Shannon Diversity was 2.06.

North Branch Blacklick Creek Downstream Redmill Discharge (NBBLCDSRMDG) located at RK 4.47 km, was sampled for macroinvertebrates in April 2015.  Habitat is optimal with a total score of 214 (Table 18A).  Field chemistry conditions of the water were:  9.9o C, Dissolved Oxygen was 9.00 mg/L, pH was 7.2 and Alkalinity was 1.0 mg/L.  Macroinvertebrates assemblage is located on Table 3.  Taxa Richness was 17 and EPT Taxa Richness was 9.  The Becks Index was 4 and the Percent Sensitive Individuals was 19.75%.  Hilsenhoff Biotic Index was 5.3, while the Shannon Diversity was 2.89.

North Branch Blacklick Creek Upstream Three Sisters Discharges (NBBLCUSTS) located at RK 0.66 km, was sampled for macroinvertebrates in April 2015.  Habitat is optimal with a total score of 212 (Table 18A).  Field chemistry conditions of the water were:  9.7o C, Dissolved Oxygen was 9.10 mg/L, pH was 7.4 and Alkalinity was 2.0 mg/L.  Macroinvertebrates assemblage is located on Table 4.  Taxa Richness was 15 and EPT Taxa Richness was 8.  The Becks Index was 6 and the Percent Sensitive Individuals was 33.33%.  Hilsenhoff Biotic Index was 4.62, while the Shannon Diversity was 2.60.

North Branch Blacklick Creek Mouth (NBBLCM) was sampled for macroinvertebrates in April 2015 and was located at RK 0.00 km.  Habitat is suboptimal with a total score of 174 (Table 18A).  Field chemistry conditions of the water were:  10.7o C, Dissolved Oxygen was 7.80 mg/L, pH was 6.3 and Alkalinity was 2.0 mg/L.  Taxa Richness was 14 and EPT Taxa Richness was 7.  The Becks Index was 8 and the Percent Sensitive Individuals was 49.02%. Hilsenhoff Biotic Index was 4.18, while the Shannon Diversity was 2.43.

Blacklick Creek Downstream Confluence of North and South Branches (BLCDSSB) was sampled for macroinvertebrates in April 2015 and was located at RK 68.33 km.  Habitat is suboptimal with a total score of 185 (Table 18A).  Field chemistry conditions of the water were:  8.4o C, Dissolved Oxygen was 10.80 mg/L, pH was 6.8 and Alkalinity was 2.0 mg/L.  Taxa Richness was 8 and EPT Taxa Richness was 4.  The Becks Index was 3 and the Percent Sensitive Individuals was 23.81%.  Hilsenhoff Biotic Index was 4.57, while the Shannon Diversity was 1.72.

Blacklick Creek Upstream Wehrum Discharge (BLCUSWDG) was sampled for macroinvertebrates in April 2015 and was located at RK 65.02 km.  Habitat is suboptimal with a total score of 196 (Table 18B).  Field chemistry conditions of the water were:  9.8o C, Dissolved Oxygen was 10.30 mg/L, pH was 6.8 and Alkalinity was 1.0 mg/L.  Taxa Richness was 8 and EPT Taxa Richness was 5.  The Becks Index was 5 and the Percent Sensitive Individuals was 47.62%.  Hilsenhoff Biotic Index was 3.81, while the Shannon Diversity was 1.75.

Blacklick Creek Downstream Wehrum Run (BLCUSWR) was sampled for macroinvertebrates in April 2015 and was located at RK 62.06 km.  Habitat is suboptimal with a total score of 159 (Table 18B).  Field chemistry conditions of the water were:  9.8o C, Dissolved Oxygen was 10.70 mg/L, pH was 6.9 and Alkalinity was 2.0 mg/L.  Taxa Richness was 8 and EPT Taxa Richness was 3.  The Becks Index was 4 and the Percent Sensitive Individuals was 22.22%. Hilsenhoff Biotic Index was 4.67, while the Shannon Diversity was 1.83.

South Branch Blacklick Creek Twin Rocks (SBBLCTR) was sampled for macroinvertebrates in November 2017 and was located at RK 8.30 km.  Habitat is suboptimal with a total score of 188 (Table 18B).  Field chemistry conditions of the water were:  0.0o C, Dissolved Oxygen was 14.6 mg/L, pH was 7.1, Conductance was 533 umhos and Alkalinity was 2.0 mg/L.  Taxa Richness was 9 and EPT Taxa Richness was 5.  The Becks Index was 2 and the Percent Sensitive Individuals was 6.48%.  Hilsenhoff Biotic Index was 5.74, while the Shannon Diversity was 1.31.

South Branch Blacklick Creek Mouth (SBBLCM) was sampled for macroinvertebrates in November 2017 and was located at RK 0.00 km.  Habitat is suboptimal with a total score of 150 (Table 18B).  Field chemistry conditions of the water were:  0.2o C, Dissolved Oxygen was 5.2 mg/L, pH was 7.51, Conductance was 558 umhos and Alkalinity was 3.0 mg/L.  Taxa Richness was 3 and EPT Taxa Richness was 2.  The Becks Index was 0 and the Percent Sensitive Individuals was 0.00%.  Hilsenhoff Biotic Index was 6.00, while the Shannon Diversity was 0.92.

Electrofishing of NBBLCUSRMDG was conducted in August 2015.  The site was 200 meters (m) in length downstream of the bridge on Red Mill Road.  Fifteen (15) species were collected during the survey (Table 11).  The fish community consisted of nearly all forage fish.  Three (3) adult trout were collected.  Two species were represented, Brook Trout (Salvelinus fontinalis) and Brown Trout (Salmo trutta).  Size distribution is listed on Table 12.  Field chemistry conditions of the water were:  17.5o C, Dissolved Oxygen was 6.72 mg/L, pH was 7.0, Conductance was 344.2 umhos and Alkalinity was 4.0 mg/L.

Electrofishing of NBBLCUSTS was conducted in August 2015.  The site was 200 m in length, approximately 500 m upstream of the Three Sisters discharge.  Thirteen (13) species were collected during the survey (Table 13).  No Trout were collected during the survey, but other sport fish were present.  Field chemistry conditions of the water were:  18.3o C, Dissolved Oxygen was 6.39 mg/L, pH was 6.7, Conductance was 329.6 umhos and Alkalinity was 3.0 mg/L.

Electrofishing of BLCUSWDG was conducted in August 2015.  The site was 200 m in length, approximately 700 m upstream of the bridge upstream of Wehrum Discharge.  Four (4) species were collected (Table 14).  No sportfish were collected at this site.  Width of the site makes it very difficult to properly shock with a backpack unit.  Other species and higher density captures species is probable.  Field chemistry conditions of the water were:  17.3o C, Dissolved Oxygen was 7.02 mg/L, pH was 6.2, Conductance was 478.0 umhos and Alkalinity was 1.0 mg/L.

Electrofishing of SBBLCTR was conducted in September 2017.  The sample reach was 200 m and started downstream of the bridge in Twin Rocks, downstream of the railroad bridge.  Four (4) species were collected (Table 15).  Two (2) Brown Trout were collected.  The size distribution is located on Table 16.  Field chemistry conditions of the water were:  15.7o C, Dissolved Oxygen was 5.20 mg/L, pH was 7.57, Conductance was 1119 umhos and Alkalinity was 2.0 mg/L.

Electrofishing of SBBLCM was conducted in September 2017.  The site started at the bridge in Vintondale.  Five (5) species were collected (Table 17).  No sportfish were collected.  Field chemistry conditions of the water were:  17.2o C, Dissolved Oxygen was 5.20 mg/L, pH was 7.15, Conductance was 952 umhos and Alkalinity was 1.0 mg/L.

Discussion

NBBLCUSRMDG site does not attain ALU with a score of 48.14 SFS (Table 21A).  Embeddeness was a major factor at the site.  Most of the rocks were large or locked together preventing proper kicking.  Elk Creek may still have a slight impairing effect, but it is possible that ALU could have been higher if an area with less boulders could have been found closer to Redmill Discharge.  Downstream below the Redmill Discharge there is relatively no change in SFS ALU. NBBLCDSRMDG does not attain ALU with a score of 48.46 SFS (Table 21A).  Most of the effects of the Redmill Discharge are physical; particulate, orange rock coating and embeddeness.  NBBLCUSTS recovers slightly from the Redmill Discharge with an ALU of 50.00 SFA (Table 21A), but the recovery is short lived.  The Three Sisters Discharge empties directly into the stream from three relief boreholes.  The ALU drops to 34.76 SFS (Table 21A).  The distance between the discharges to the confluence with South Branch is very short and proper mixing through the entire stream width is not possible.  This sample was a composite of two (2) different water qualities.  The South Branch site near Twin Rocks, does not attain ALU with a score of 27.50 SFS (Table 21B).  The macroinvertebrate composition of the site was large with 216 macros picked.  The site unfortunately had low Taxa Richness.  The genera that is present are present in high numbers.  SBBLCM also does not attain SFS ALU with a score of 16.83.  The last three sites on Blacklick Creek, two above the Wehrum Discharge and one downstream, all do not attain SFS ALU (see Tables 21A and 21B).

Fish composition at all sites on the North Branch and South Branch were typical of AMD streams that are periodically affected by AMD discharges.  In all cases there was a strong forage and non-sport fish presence.  There were a few sport fish present.  In most cases, the volume of water present was not practical for a small backpack electrofisher.

Blacklick Creek Watershed has a cycle of AMD pollution affecting it from its headwaters through its length past Two-Lick Creek.  Many areas have seen some minor to moderate recovery while others remain heavily impacted from AMD.  The majority of the AMD pollution is in the middle of the watershed near Vintondale.  The current project planned for an active treatment plant will tie several mine pools into one and process the water near the junction of the North and South Branches of Blacklick Creek.  This project will be a major boon for water quality in central PA.  The processed water will significantly reduce both metals and acidity.  The project could also be critical in reducing sedimentation in the Conemaugh Lake Reservoir.

Appendix

Table 1.  Social Data for North and South Branch Blacklick Creek and Upper Blacklick Creek Watershed.

| USGS Watershed Code: | 05010007 |
| --- | --- |
| USGS Stream Code: | 000137 |
| PA DEP Chapter 93 Code: | North Branch and South Branch: Cold Water Fishery, Main Stem: Trout Stocked Fishery |
| Counties/Municipalities: | Cambria and Indiana Counties/Cambria, Jackson, Blacklick, East Wheatfield and Buffington Townships/Nanty Glo and Vintondale Boroughs |
| Topo Quad: | Carrolltown, Ebensburg, Nanty Glo, Colver, Strongstown and Vintondale |

Table 2.  Raw Macroinvertebrate Data for North Branch Blacklick Creek Upstream Redmill Discharge (NBBLCUSRMDG) Sampled April 15, 2015.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Ephemeroptera | Maccaffertium | 5 |
|  | Baetis | 12 |
| Plecoptera | Acroneuria | 1 |
|  | Leuctra | 24 |
|  | Amphinemura | 8 |
|  | Taenionema | 3 |
| Trichoptera | Chimarra | 4 |
|  | Cheumatopsyche | 2 |
|  | Hydropsyche | 13 |
| Diptera | Tipula | 1 |
|  | Chironomidae | 44 |
| Megaloptera | Nigronia | 1 |
| Odonata | Lanthus | 1 |
| Oligocheata | Oligocheata | 8 |
| Decapoda | Cambaridae | 1 |
| Sum of all Individuals |  | 128 |
| Number Sub-samples picked out of 24 |  | 24 |

Table 3.  Raw Macroinvertebrate Data for North Branch Blacklick Creek Downstream Redmill Discharge (NBBLCDSRMDG) Sampled April 16, 2015.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Ephemeroptera | Maccaffertium | 7 |
|  | Isonychia | 2 |
|  | Baetisca | 1 |
|  | Baetis | 4 |
|  | Caenis | 2 |
| Plecoptera | Leuctra | 2 |
|  | Taenionema | 1 |
| Trichoptera | Chimarra | 2 |
|  | Hydropsyche | 12 |
| Diptera | Antocha | 3 |
|  | Ceratopogon | 2 |
|  | Simulium | 1 |
|  | Chironomidae | 30 |
| Coleoptera | Optioservus | 5 |
| Megaloptera | Nigronia | 1 |
| Oligocheata | Oligocheata | 3 |
| Decapoda | Cambaridae | 3 |
| Sum of all Individuals |  | 81 |
| Number Sub-samples picked out of 24 |  | 24 |

Table 4.  Raw Macroinvertebrate Data for North Branch Blacklick Creek Upstream Three Sisters Discharge (NBBLCUSTS) Sampled April 16, 2015.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Ephemeroptera | Maccaffertium | 4 |
|  | Caenis | 1 |
| Plecoptera | Leuctra | 2 |
|  | Eccoptura | 1 |
|  | Taenionema | 5 |
| Trichoptera | Rhyacophila | 1 |
|  | Chimarra | 1 |
|  | Hydropsyche | 7 |
| Diptera | Antocha | 1 |
|  | Hemerodromia | 1 |
|  | Twinnia | 1 |
|  | Chironomidae | 7 |
| Coleoptera | Optioservus | 7 |
| Oligocheata | Oligocheata | 2 |
| Decapoda | Cambaridae | 1 |
| Sum of all Individuals |  | 42 |
| Number Sub-samples picked out of 24 |  | 24 |

Table 5.  Raw Macroinvertebrate Data for North Branch Blacklick Creek Mouth (NBBLCM) Sampled April 16, 2015.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Ephemeroptera | Paraleptophlebia | 1 |
|  | Ephemera | 1 |
|  | Maccaffertium | 14 |
|  | Isonychia | 2 |
|  | Caenis | 4 |
| Plecoptera | Leuctra | 2 |
| Trichoptera | Hydropsyche | 4 |
| Diptera | Prosimulium | 2 |
|  | Tipula | 2 |
|  | Molophilus | 1 |
|  | Chironomidae | 12 |
| Megaloptera | Nigronia | 3 |
|  | Sialis | 1 |
| Odonata | Lanthus | 2 |
| Sum of all Individuals |  | 51 |
| Number Sub-samples picked out of 24 |  | 24 |

Table 6. Raw Macroinvertebrate Data for Blacklick Creek Downstream South Branch Blacklick Creek (BLCDSSB) Sampled April 28, 2015.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Ephemeroptera | Maccaffertium | 2 |
|  | Baetis | 1 |
| Plecoptera | Leuctra | 3 |
| Trichoptera | Chimarra | 3 |
| Diptera | Tipula | 1 |
|  | Molophilus | 1 |
|  | Chironomidae | 9 |
| Oligocheata | Oligocheata | 1 |
| Sum of all Individuals |  | 21 |
| Number Sub-samples picked out of 24 |  | 24 |

Table 7.  Raw Macroinvertebrate Data for Blacklick Creek Upstream Wehrum Discharge (BLCUSWDG) Sampled April 28, 2015.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Ephemeroptera | Maccaffertium | 2 |
| Plecoptera | Leuctra | 4 |
|  | Taenionema | 3 |
| Trichoptera | Rhyacophila | 1 |
|  | Hydropsyche | 1 |
| Diptera | Stilobezzia | 1 |
|  | Chironomidae | 8 |
| Coleoptera | Optioservus | 1 |
| Sum of all Individuals |  | 21 |
| Number Sub-samples picked out of 24 |  | 24 |

Table 8.  Raw Macroinvertebrate Data for Blacklick Creek Downstream Wehrum Run (BLCDSWR) Sampled April 28, 2015.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Plecoptera | Leuctra | 3 |
| Trichoptera | Chimarra | 1 |
|  | Hydropsyche | 5 |
| Diptera | Ceratopogon | 3 |
|  | Chelifera | 1 |
|  | Hemerodromia | 2 |
|  | Chironomidae | 2 |
| Megaloptera | Nigronia | 1 |
| Sum of all Individuals |  | 18 |
| Number Sub-samples picked out of 24 |  | 24 |

Table 9.  Raw Macroinvertebrate Data for South Branch Blacklick Creek Twin Rocks, PA (SBBLCTR) Sampled December 15, 2017.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Ephemeroptera | Maccaffertium | 10 |
|  | Baetis | 31 |
| Trichoptera | Rhyacophila | 4 |
|  | Cheumatopsyche | 10 |
|  | Hydropsyche | 129 |
| Diptera | Chelifera | 1 |
|  | Chironomidae | 27 |
|  | Hemerodromia | 1 |
| Grammaridae | Gammarus | 3 |
| Sum of all Individuals |  | 216 |
| Number Sub-samples picked out of 24 |  | 9 |

Table 10.  Raw Macroinvertebrate Data for South Branch Blacklick Creek Mouth (SBBLCM) Sampled December 15, 2017.

| Taxa Group | Taxa ID Level | Number of Individuals |
| --- | --- | --- |
| Trichoptera | Cheumatopsyche | 2 |
|  | Hydropsyche | 8 |
| Diptera | Chironomidae | 12 |
| Sum of all Individuals |  | 22 |
| Number Sub-samples picked out of 24 |  | 24 |

Table 11.  Raw Fish Data for North Branch Blacklick Creek Upstream Redmill Discharge (NBBLCUSRMDG) Sampled August 7, 2015.

| Common Name | Number |
| --- | --- |
| Brook Trout | 2 |
| Brown Trout | 1 |
| Bluegill | 2 |
| Pumpkinseed | 5 |
| Fantail Darter | 4 |
| Greenside Darter | 5 |
| Johnny Darter | 13 |
| Blackside Darter | 2 |
| White Sucker | 21 |
| Mottled Sculpin | 29 |
| Creek Chub | 8 |
| Blacknose Dace | 130 |
| Longnose Dace | 3 |
| Central Stoneroller | 2 |
| Common Shiner | 3 |

Table 12.  Size Distribution of Trout by 25 mm Group and Species for North Branch Blacklick Creek Upstream Redmill Discharge (NBBLCUSRMDG) Sampled August 7, 2015.

| Size Distribution of Trout by 25 mm group |  |
| --- | --- |
| 250 | 2-Brook Trout |
| 250 | 1-Brown Trout |

Table 13.  Size Distribution of Trout by 25 mm Group and Species for North Branch Blacklick Creek Upstream Three Sisters Discharge (NBBLCUSTS) Sampled August 8, 2015.

| Common Name | Number |
| --- | --- |
| Chain Pickerel | 2 |
| Rock Bass | 4 |
| Bluegill | 21 |
| Pumpkinseed | 7 |
| Fantail Darter | 14 |
| Johnny Darter | 11 |
| Blackside Darter | 2 |
| White Sucker | 51 |
| Mottled Sculpin | 20 |
| Creek Chub | 27 |
| Blacknose Dace | 221 |
| Central Stoneroller | 8 |
| Flathead Minnow | 30 |

Table 14.  Raw Fish Data for Blacklick Creek Upstream Wehrum Discharge (BLCUSWDG) Sampled August 8, 2015.

| Common Name | Number |
| --- | --- |
| White Sucker | 1 |
| Mottled Sculpin | 1 |
| Creek Chub | 15 |
| Blacknose Dace | 56 |

Table 15.  Raw Fish Data for South Branch Blacklick Creek Twin Rocks, PA (SBBLCTR) September 21, 2017.

| Common Name | Number |
| --- | --- |
| Brook Trout | 2 |
| Johnny Darter | 9 |
| White Sucker | 13 |
| Blacknose Dace | 47 |

Table 16.  Size Distribution of Brown Trout for Blacklick Creek Twin Rocks, PA (SBBLCTR) Sampled September 21, 2017.

| Size Distribution of Brown Trout by 25 mm group |  |
| --- | --- |
| 250 | 1 |
| 375 | 1 |

Table 17.  Raw Fish Data for South Branch Blacklick Creek Mouth (SBBLCM) Sampled September 21, 2017.

| Common Name | Number |
| --- | --- |
| Central Stoneroller | 2 |
| Common Shiner | 4 |
| Creek Chub | 4 |
| White Sucker | 10 |
| Blacknose Dace | 17 |

Tables 18 A-B.  Habitat Assessment Parameters and Scores; Arranged from Headwater (Left) to Mouth and Receiving Water (Right), for Upper Blacklick Creek taken April 2015.

Table 18A:

| Habitat Assessment Parameters | NBBLCUSRMDG | NBBLCDSRMDG | NBBLCUSTS | NBBLCM | BLCDSSB |
| --- | --- | --- | --- | --- | --- |
| Instream Cover | 18 | 17 | 17 | 18 | 17 |
| Epifaunal Substrate | 20 | 16 | 16 | 16 | 18 |
| Embeddedness | 19 | 15 | 12 | 6 | 15 |
| Velocity/Depth Regimes | 20 | 19 | 16 | 19 | 14 |
| Channel Alterations | 17 | 20 | 20 | 8 | 17 |
| Sediment Deposition | 18 | 17 | 17 | 14 | 17 |
| Frequency of Riffles | 20 | 16 | 19 | 18 | 17 |
| Channel Flow Status | 20 | 19 | 20 | 20 | 19 |
| Condition of Banks | 18 | 17 | 17 | 16 | 11 |
| Bank Vegetative Protection | 19 | 18 | 19 | 16 | 12 |
| Grazing or Other Disruptive Pressures | 18 | 20 | 20 | 12 | 15 |
| Riparian Vegetative Zone Width | 16 | 20 | 19 | 11 | 13 |
| Total | 223 | 214 | 212 | 174 | 185 |

Table 18B:

| Habitat Assessment Parameters | BLCUSWDG | BLCUSWR | SBBLCTR | SBBLCM |
| --- | --- | --- | --- | --- |
| Instream Cover | 18 | 13 | 17 | 16 |
| Epifaunal Substrate | 19 | 15 | 16 | 13 |
| Embeddedness | 7 | 6 | 10 | 13 |
| Velocity/Depth Regimes | 18 | 10 | 17 | 10 |
| Channel Alterations | 19 | 16 | 15 | 10 |
| Sediment Deposition | 14 | 8 | 12 | 16 |
| Frequency of Riffles | 16 | 12 | 17 | 17 |
| Channel Flow Status | 18 | 19 | 19 | 18 |
| Condition of Banks | 15 | 14 | 14 | 19 |
| Bank Vegatative Protection | 18 | 18 | 19 | 8 |
| Grazing or Other Disruptive Pressures | 18 | 16 | 16 | 6 |
| Riparian Vegetative Zone Width | 16 | 12 | 16 | 4 |
| Total | 196 | 159 | 188 | 150 |

Tables 19 A-B.  Field Water Quality Measurements; Arranged from Headwater (Left) to Mouth and Receiving Water (Right), for Upper Blacklick Creek taken April and August 2016.

Table 19A:

|  | NBBLCUSRMDG |  | NBBLCDSRMDG |  | NBBLCUSTS |  | NBBLCM |  | BLCDSSB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water Quality | Macro | Fish | Macro | Fish | Macro | Fish | Macro | Fish | Macro | Fish |
| Temperature (o C) | 9.3 | 17.5 | 9.9 | - | 9.7 | 18.3 | 10.7 | - | 8.4 | - |
| Dissolved Oxygen (mg/L) | 10.00 | 6.72 | 9.00 | - | 9.10 | 6.39 | 7.80 | - | 10.80 | - |
| pH | 7.3 | 7.0 | 7.2 | - | 7.4 | 6.7 | 6.3 | - | 6.8 | - |
| Conductance (umhos) | - | 344.2 | - | - | - | 329.6 | - | - | - | - |
| Alkalinity (mg/L) | 1.0 | 4.0 | 1.0 | - | 2.0 | 3.0 | 2.0 | - | 2.0 | - |

Table 19B:

|  | BLCUSWDG |  | BLCUSWR |  | SBBLCTR |  | SBBLCM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Water Quality | Macro | Fish | Macro | Fish | Macro | Fish | Macro | Fish |
| Temperature (o C) | 9.8 | 17.3 | 9.8 | - | 0.0 | 15.7 | 0.2 | 17.2 |
| Dissolved Oxygen (mg/L) | 10.30 | 7.02 | 10.70 | - | 14.6 | 5.20 | 5.2 | 5.20 |
| pH | 6.8 | 6.2 | 6.9 | - | 7.1 | 7.57 | 7.51 | 7.15 |
| Conductance (umhos) | - | 478.0 | - | - | 533 | 1119 | 558 | 952 |
| Alkalinity (mg/L) | 1.0 | 1.0 | 2.0 | - | 2.0 | 2.0 | 3.0 | 1.0 |

Tables 20 A-B.  Macroinvertebrate IBI Scores; Arranged from Headwater (Left) to Mouth and Receiving Water (Right), for Upper Blacklick Creek taken April 2015.

Table 20A:

| Macroinvertebrate IBI scores | NBBLCUSRMDG | NBBLCDSRMDG | NBBLCUSTS | NBBLCM | BLCDSSB |
| --- | --- | --- | --- | --- | --- |
| Taxa Richness | 15 | 17 | 15 | 14 | 8 |
| EPT Taxa Richness | 9 | 9 | 8 | 7 | 4 |
| Becks Index | 7 | 4 | 6 | 8 | 3 |
| Hilsenhoff Biotic Index | 4.59 | 5.3 | 4.62 | 4.18 | 4.57 |
| Shannon Diversity | 2.06 | 2.89 | 2.60 | 2.43 | 1.72 |
| Percent Sensitive PTV 0-3 Individuals | 32.81 | 19.75 | 33.33 | 49.02 | 23.81 |

Table 20B:

| Macroinvertebrate IBI scores | BLCUSWDG | BLCUSWR | SBBLCTR | SBBLCM |
| --- | --- | --- | --- | --- |
| Taxa Richness | 8 | 8 | 9 | 3 |
| EPT Taxa Richness | 5 | 3 | 5 | 2 |
| Becks Index | 5 | 4 | 2 | 0 |
| Hilsenhoff Biotic Index | 3.81 | 4.67 | 5.74 | 6.00 |
| Shannon Diversity | 1.75 | 1.83 | 1.31 | 0.92 |
| Percent Sensitive PTV 0-3 Individuals | 47.62 | 22.22 | 6.48 | 0.00 |

Tables 21 A-B.  IBI Standardizations and Aquatic Life Use (ALU) Benchmark; Arranged from Headwater (Left) to Mouth and Receiving Water (Right), for Upper Blacklick Creek taken April 2015.

Table 21A:

|  | NBBLCUSRMDG |  | NBBLCDSRMDG |  | NBBLCUSTS |  | NBBLCM |  | BLCDSSB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IBI Standardization and Aquatic Life Use Benchmark | SFS | LFS | SFS | LFS | SFS | LFS | SFS | LFS | SFS | LFS |
| Taxa Richness | 0.45 | 0.48 | 0.52 | 0.55 | 0.45 | 0.48 | 0.42 | 0.45 | 0.24 | 0.26 |
| EPT Taxa Richness | 0.47 | 0.56 | 0.47 | 0.56 | 0.42 | 0.50 | 0.37 | 0.44 | 0.21 | 0.25 |
| Becks Index | 0.18 | 0.32 | 0.11 | 0.18 | 0.16 | 0.27 | 0.21 | 0.36 | 0.08 | 0.14 |
| Hilsenhoff Biotic Index | 0.67 | 0.78 | 0.58 | 0.68 | 0.66 | 0.77 | 0.72 | 0.84 | 0.67 | 0.78 |
| Shannon Diversity | 0.72 | 0.72 | 1.00 | 1.00 | 0.91 | 0.91 | 0.85 | 0.85 | 0.60 | 0.60 |
| Percent Sensitive PTV 0-3 Individuals (PSI) | 0.39 | 0.49 | 0.23 | 0.30 | 0.39 | 0.50 | 0.58 | 0.73 | 0.28 | 0.36 |
| ALU Benchmark | 48.14 | 55.92 | 48.46 | 54.43 | 50.00 | 57.32 | 52.51 | 61.25 | 34.76 | 39.75 |

Table 21B:

|  | BLCUSWDG |  | BLCDSWR |  | SBBLCTR |  | SBBLCM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IBI Standardization and Aquatic Life Use Benchmark | SFS | LFS | SFS | LFS | SFS | LFS | SFS | LFS |
| Taxa Richness | 0.24 | 0.26 | 0.24 | 0.26 | 0.27 | 0.29 | 0.09 | 0.1 |
| EPT Taxa Richness | 0.26 | 0.31 | 0.16 | 0.19 | 0.26 | 0.31 | 0.11 | 0.13 |
| Becks Index | 0.13 | 0.23 | 0.11 | 0.18 | 0.05 | 0.09 | 0.00 | 0.00 |
| Hilsenhoff Biotic Index | 0.76 | 0.89 | 0.66 | 0.77 | 0.53 | 0.61 | 0.49 | 0.58 |
| Shannon Diversity | 0.61 | 0.61 | 0.64 | 0.64 | 0.46 | 0.46 | 0.32 | 0.32 |
| Percent Sensitive PTV 0-3 Individuals (PSI) | 0.56 | 0.71 | 0.26 | 0.33 | 0.08 | 0.10 | 0.00 | 0.00 |
| ALU Benchmark | 42.92 | 50.22 | 34.46 | 39.49 | 27.50 | 31.05 | 16.83 | 18.63 |

Table 22.  Sampling Point Name, Abbreviation and Type of Sampling Completed at each Point with Longitude and Latitude.

| Sample Point Name | ID | Macro | Fish | Longitude (West) | Latitude (North) | River Kilometer |
| --- | --- | --- | --- | --- | --- | --- |
| North Branch Blacklick Creek Upstream Redmill Discharge | NBBLCUSRMDG | x | x | -78.9036 | 41.5125 | 5.75 |
| North Branch Blacklick Creek Downstream Redmill Discharge | NBBLCDSRMDG | x | - | -78.9049 | 40.5048 | 4.47 |
| North Branch Blacklick Creek Upstream Three Sisters Discharge | NBBLCUSTS | x | x | -78.9205 | 40.4874 | 0.66 |
| North Branch Blacklick Creek Mouth | NBBLCM | x | x | -78.9238 | 40.4840 | 0.00 |
| Blacklick Creek Downstream Confluence North and South Branches | BLCDSSB | x | - | -78.9311 | 40.4817 | 68.33 |
| Blacklick Creek Upstream Wehrum Discharge | BLCUSWDG | x | x | -78.9465 | 40.4703 | 65.02 |
| Blacklick Creek Downstream Wehrum Run | BLCDSWR | x | - | -78.9238 | 40.4837 | 62.06 |
| South Branch Blacklick Creek Twin Rocks, PA | SBBLCTR | x | x | -78.8628 | 40.4904 | 8.30 |
| South Branch Blacklick Creek Mouth | SBBLCM | x | x | -78.9238 | 40.4837 | 0.00 |

Figure 1.  Map of Blacklick Creek Sampling Points

References

Allen, M. (2006).  Glossary of Aquatic Insect Morphology.  Prepared for the Caddisfly Press.

Cooper, E.L. (1983).  Fishes of Pennsylvania and Northeastern United States.  Pennsylvania State University.

Diana, J.S. (2004).  Biology and Ecology of Fishes, 2nd Edition.

Gelhaus, J.K. (2008).  Manual for the Identification Aquatic Cranefly Larvae of North America.  Prepared for the North American Benthological Society Workshop, Salt Lake City, Utah.

Marcinko, et al. (1986).  Procedures for Stream and River Inventory Information Input. Pennsylvania Fish and Boat Commission, Division of Fisheries Management, Area 8, Somerset, Pennsylvania.

Merritt, R.W., et al. (2008).  An Introduction to the Aquatic Insects of North America, 4th Edition.

Murphy, B.R. and Willis, D.W. Editors. (1996).  Fisheries Techniques, 2nd Edition.  American Fisheries Society, Bethesda, Maryland.

Peckarsky, B.L., et al. (1990).  Freshwater Macroinvertebrates of Northeastern North America.

Pennsylvania Department of Environmental Protection. (1999).  Department of Environmental Protection, Chapter 93.  Pennsylvania Department of Environmental Protection, Bureau of Water Quality, Harrisburg, Pennsylvania.

Pennsylvania Department of Environmental Protection. (2013).  Instream Comprehensive Evaluation Survey.  Pennsylvania Department of Environmental Protection, Bureau of Water Quality, Harrisburg, Pennsylvania.

Pfeiffer, J., et al. (2006).  A Photographic Key to the Beatidae of EPA Region Three.  Prepared by Ecoanalysts, Inc. for the United States Environmental Protection Agency, Office of Environmental Information Analysis Division, Washington, D.C.

Pfeiffer, J., et al. (2006).  A Photographic Key to the Simuliidae of EPA Region Three.  Prepared by Ecoanalysts, Inc. for the United States Environmental Protection Agency, Office of Environmental Information Analysis Division, Washington, D.C.

Pfeiffer, J., et al. (2006).  Separation of the Larvae of the Stonefly Families Leuctridae and Capniidae.  Prepared by Ecoanalysts, Inc. for the United States Environmental Protection Agency, Office of Environmental Information Analysis Division, Washington, D.C.

References

(Continued)

Raleigh, R.F. (1982).  Habitat Suitability Index Models:  Brook Trout.  U.S. Department of Interior, Fish and Wildlife Service.  FWS/OBS-82/10.24. 42 pp.

Stewart, K.W. and Stark, B.P. (2002).  Nymphs of North American Stonefly Genera (Plecoptera), 2nd Edition.

Wiggins, G.B. (1998).  Larvae of the North American Caddisfly Genera (Tricoptera), 2nd Edition.