April 9, 2025

Aulds Run Virginia Mine # 14

Observations and Field Water Parameters Assessment

| Id Flag | Location Description | pH | uS/cm | Comments |
| --- | --- | --- | --- | --- |
| A | AR @ Rhine Road | 6.70 | 278 | 30 yards below Rhine Road Culvert. Stream team sample point 629 |
| B | AR above VA # 14 | 7.12 | 283 | AR 50 yards upstream of VA 14 discharge; below C tributary |
| C | UN tributary | 7.11 | 575 | Tributary flows into AR above AR. Good PH whole length |
| D | BLC @ GTT upstream of AR discharge | 6.95 | 182 | Measurements taken near edge of shoreline |
| E | BLC @ GTT downstream of AR discharge | 6.71 | 200 | Measurements taken near edge of shoreline |
| F | AR above GTT and waterfall and culvert | 3.51 | 451 | Stream team sample point 627 |
| G | Discharge across path above sample point F unnamed tributary | 3.49 | 649 | Stream team sample point 628; lower acidity readings and extra flow and orange color compared to previous Stream Team samples |
| H | AR 50 feet upstream of unnamed tributary sample point G | 4.47 | 371 | Aulds Run stream sample |
| I | 70 yards above H on AR | 4.43 | 366 | Aulds Run stream sample |
| J | 200 yards above I on AR | 4.32 | 356 | Aulds Run Stream sample |
| K | AR upstream of explosives building | 3.75 | 391 | Aulds Run stream sample |
| L | Unnamed tributary or discharge near old mine structure | 3.23 | 175 | Unnamed tributary or seep that flows into Aulds Run |
| M | Virginia 14 discharge | 2.51 | 1250 | Sample taken @ end of discharge area after water flows over land; stream team sample point 626 |

GTT = Ghost Town Trail Indiana County Parks and Recreation Rail trail

AR = Aulds Run

BLC = Blacklick Creek